RASUES Security analysis

| Sniffing |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Where | Threat | Consequences | CIA | Attacker | Property | Mitigation |
| Phase 1 |  |  |  |  |  |  |  |  |
| 1 |  | Attester ←all→ Relying party | Attacker may attempt to listen to the communication. | The communication authenticity is broken among all entities | Confidentiality | AN(Netowrk) | FS4 (Integrity Assurance) | end-to-end encryption (TLS) |
| 2 |  | Attester ← Relying party | Attacker may attempt to listen to the communication. | Learning when the AWDT timer is initialized | Confidentiality | AN(Netowrk) | FS4 (Integrity Assurance) FS2 (Recoverability) | end-to-end encryption (TLS) |
| 3 |  | Target environment → Attestation environment | Attacker may attempt to listen to the communication. | Learning when the AWDT timer is invoked | Confidentiality | AN(Netowrk) | FS4 (Integrity Assurance), FS2 (Recoverability) | end-to-end encryption (TLS) |
| 4 |  | RVP ← Relying party | Attacker may attempt to listen to the communication. | Learning when the AWDT timer is started | Confidentiality | AN(Netowrk) | FS4 (Integrity Assurance) | end-to-end encryption (TLS) |
| 5 |  | Verifier ← RVP | Attacker may attempt to listen to the communication. | Learning the derived expected state in the whitelist | Confidentiality | AN(Netowrk) | FS4 (Integrity Assurance), FS1 (Updateability) | end-to-end encryption (TLS) |
| 6 |  | Attester ← RVP | Attacker may attempt to listen to the communication. | Learning the content of the updated transmission | Confidentiality | AN(Netowrk) | FS4 (Integrity Assurance), FS1 (Updateability) | end-to-end encryption (TLS) |
| 7 |  | Target environment → Attestation environment (2) | Attacker may attempt to listen to the communication. | Learning the content of the update during the update | Confidentiality | AN(Netowrk) | FS4 (Integrity Assurance) FS1 (Updateability) | end-to-end encryption (TLS) |
| Phase 2 |  |  |  |  |  |  |  |  |
| 8 |  | Attester ← Verifier | Attacker may attempt to listen to the communication. | Learning the content of the attestation request | Confidentiality | AN(Netowrk) | FS4 (Integrity Assurance), FS3 (Freshness) | end-to-end encryption (TLS) |
| 9 |  | Target environment → Attestation environment (3) | Attacker may attempt to listen to the communication. | Learning the integrity measurement process | Confidentiality | AN(Netowrk) | FS4 (Integrity Assurance), FS3 (Freshness) | end-to-end encryption (TLS) |
| 10 |  | Target environment ← Attestation environment | Attacker may attempt to listen to the communication. | Learning the integrity measurement is completed | Confidentiality | AN(Netowrk) | FS4 (Integrity Assurance), FS3 (Freshness) | end-to-end encryption (TLS) |
| 11 |  | Attester → Vérifier | Attacker may attempt to listen to the communication. | Learning the evidence through the attestation response, thus results in reducing its trustworthiness | Confidentiality | AN(Netowrk) | FS4 (Integrity Assurance), FS3 (Freshness) | end-to-end encryption (TLS) |
| Phase 3 |  |  |  |  |  |  |  |  |
| 12 |  | Attester ← Verifier (2) | Attacker may attempt to listen to the communication. | Learning the attestation result | Confidentiality | AN(Netowrk) | FS4 (Integrity Assurance), FS3 (Freshness) | end-to-end encryption (TLS) |
| 13 |  | Attester → Relying party | Attacker may attempt to listen to the communication. | Learning the attestation result | Confidentiality | AN(Netowrk) | FS4 (Integrity Assurance), FS3 (Freshness) | end-to-end encryption (TLS) |
| 14 |  | Attester ← Relying party (2) | Attacker may attempt to listen to the communication. | Learning the decision made by the relying party and knowing the deferral tickets | Confidentiality | AN(Netowrk) | FS4 (Integrity Assurance), FS3 (Freshness) | end-to-end encryption (TLS) |
| 15 |  | Target environment → Attestation environment (4) | Attacker may attempt to listen to the communication. | Learning the decision made by the relying party and knowing the deferral tickets | Confidentiality | AN(Netowrk) | FS4 (Integrity Assurance), FS2 (Recoverability) | end-to-end encryption (TLS) |
| Blocking |  |  |  |  |  |  |  |  |
|  |  | Where | Threat | Consequences | CIA | Attacker | Property | Mitigation |
| Phase 1 |  |  |  |  |  |  |  |  |
| 1 |  | Attester ←all→ Relying party | Attacker may attempt to block access to the entity. | Preventing entities from communicating with each other | Availability | AN(Netowrk) | FS4 (Integrity Assurance) | TLS can detect violations |
| 2 |  | Attester ← Relying party | Attacker may attempt to block access to the entity. | Blocking the instruction of setting up the AWDT | Availability | AN(Netowrk) AS(Software) | FS4 (Integrity Assurance) FS2 (Recoverability) | We assumed to be trusted |
| 3 |  | Target environment → Attestation environment | Attacker may attempt to block access to the entity. | Blocking invoking AWDT | Availability | AN(Netowrk) | FS4 (Integrity Assurance), FS2 (Recoverability) | TPM2 in the attestation environment |
| 4 |  | RVP ← Relying party | Attacker may attempt to block access to the entity. | Blocking starting AWDT | Availability | AN(Netowrk) | FS4 (Integrity Assurance) | end-to-end encryption (TLS) |
| 5 |  | Verifier ← RVP | Attacker may attempt to block access to the entity. | Blocking deriving the expected state from the whitelist | Availability | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS1 (Updateability) | end-to-end encryption (TLS) |
| 6 |  | Attester ← RVP | Attacker may attempt to block access to the entity. | The update will not be delivered due to the provider RVP being blocked. Thus, the attester will not be updated and will remain in its current state, either vulnerable or benign | Availability | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS1 (Updateability) | RVP is assumed to be trusted Since the timer has been initiated recently, an attack will be detected when the watchdog has timed out |
| 7 |  | Target environment → Attestation environment (2) | Attacker may attempt to block access to the entity. | Blocking triggering update at the measurement. | Availability | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance) FS1 (Updateability) | TPM2 in the attestation environment |
| Phase 2 |  |  |  |  |  |  |  |  |
| 8 |  | Attester ← Verifier | Attacker may attempt to block access to the entity. | Verifier not capable of initiating attestation request, thus cannot determine the trustworthiness of attester’s expected state | Availability | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | end-to-end encryption (TLS), IMA, cryptographic key exchange |
| 9 |  | Target environment → Attestation environment (3) | Attacker may attempt to block access to the entity. | The integrity measurement will not be triggered in the attestation environment | Availability | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | IMA, cryptographic key exchange, TPM |
| 10 |  | Target environment ← Attestation environment | Attacker may attempt to block access to the entity. | The attestation environment will not provide the evidence of the measurement | Availability | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | It will fail to start the measurement. It will wait until the watchdog timer has rolled out, IMA, and key exchange |
| 11 |  | Attester → Vérifier | Attacker may attempt to block access to the entity. | The attester is unavailable to comply with the required measurement and thus cannot continue with verification | Availability | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | Attestation will time out; thus, it will not be verified It will wait until the watchdog timer has rolled out. IMA and key exchange |
| Phase 3 |  |  |  |  |  |  |  |  |
| 12 |  | Attester ← Verifier (2) | Attacker may attempt to block access to the entity. | The effect is minimal, but the verifier cannot send the verified evidence to the attester | Availability | AN(Netowrk),AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | IMA and key exchange |
| 13 |  | Attester → Relying party | Attacker may attempt to block access to the entity. | The Appraisal process will fail due to the relying party not receiving the attestation result. Thus, the attester might be malicious or vulnerable The device will not be able to receive any further updates and will become completely unavailable for updates or further assessments, or verification | Availability | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | Out of scope. The device will be rest, however, if the device is always being rest due to an attack, this leads to an endless loop, thus cannot be mitigated, since this is a type of DoS |
| 14 |  | Attester ← Relying party (2) | Attacker may attempt to block access to the entity. | Attester cannot retrieve the Deferral Ticket, indication of an attack. Denying access to the attester, thus the relying party cannot make a decision, leaving relying party unaware of the current status of the attester even though its expected state has been verified | Availability | AN(Netowrk),AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | AWDT |
| 15 |  | Target environment → Attestation environment (4) | Attacker may attempt to listen to the communication. |  | Availability | AN(Netowrk),AS(Software) | FS4 (Integrity Assurance), FS2 (Recoverability) | AWDT |
| Modification |  |  |  |  |  |  |  |  |
|  |  | Where | Threat | Consequences | CIA | Attacker | Property | Mitigation |
| Phase 1 |  |  |  |  |  |  |  |  |
| 1 |  | Attester ←all→ Relying party | Attacker may attempt to modify the content of all the entities. | Leads to the impersonation of another malicious device | Integrity | AN(Netowrk), AP(Priviledged) | FS4 (Integrity Assurance) | end-to-end encryption (TLS) |
| 2 |  | Attester ← Relying party | Attacker may attempt to modify the content of the entity. | Attacker modifying the AWDT timer. Preventing the attester from resetting if it is malicious | Integrity | AN(Netowrk), AS(Software), AP(Priviledged) | FS4 (Integrity Assurance), FS2 (Recoverability) | The relying party is using the public cryptographic key of the attester to communicate with the attester in the attestation environment |
| 3 |  | Target environment → Attestation environment | Attacker may attempt to modify the content of the entity. | Modifying the invoke request, so the attestation environment might be invoked when it is not supposed to be | Integrity | AN(Netowrk), AS(Software), AP(Priviledged) | FS4 (Integrity Assurance), FS2 (Recoverability) | Cryptographic key exchange, TPM end-to-end encryption (TLS) |
| 4 |  | RVP ← Relying party | Attacker may attempt to modify the content of the entity. | Modifying the content of the AWDT so that the attacker misconfigures the initialization time | Integrity | AN(Netowrk), AS(Software), AP(Priviledged) | FS4 (Integrity Assurance) | Cryptographic key exchange, AWDT, end-to-end encryption (TLS) |
| 5 |  | Verifier ← RVP | Attacker may attempt to modify the content of the entity. | Modifying the whitelist | Integrity | AN(Netowrk), AS(Software), AP(Priviledged) | FS4 (Integrity Assurance), FS1 (Updateability) | end-to-end encryption (TLS), AWDT, IMA |
| 6 |  | Attester ← RVP | Attacker may attempt to modify the content of the entity. | Modifying the supplied update before installation | Integrity | AN(Netowrk), AS(Software), AP(Priviledged) | FS4 (Integrity Assurance), FS1 (Updateability) | end-to-end encryption (TLS), TPM |
| 7 |  | Target environment → Attestation environment (2) | Attacker may attempt to modify the content of the entity. | Modifying how the attestation environment or triggering measurement by an unintended entity | Integrity | AN(Netowrk), AS(Software), AP(Priviledged) | FS4 (Integrity Assurance), FS1 (Updateability) | There is a TPM in the attestation environment and TPM |
| Phase 2 |  |  |  |  |  |  |  |  |
| 8 |  | Attester ← Verifier | Attacker may attempt to modify the content of the entity. | Invoking untrusted or modified attestation requests. Unnecessary attestation request challenge | Integrity | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | end-to-end encryption (TLS), IMA, TPM |
| 9 |  | Target environment → Attestation environment (3) | Attacker may attempt to modify the content of the entity. | Modifying when triggering the attestation measurement | Integrity | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | Cryptographic key exchange, TPM, IMA |
| 10 |  | Target environment ← Attestation environment | Attacker may attempt to modify the content of the entity. | Modifying the integrity measurement of attestation | Integrity | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | Cryptographic key exchange, TPM, IMA |
| 11 |  | Attester → Verifier | Attacker may attempt to modify the content of the entity. | Modifying the evidence. Verifying wrong evidence or wrong Device Resulting in an attestation to be legitimate Note: what is included in the evidence, such as the device ID. Device ID, Nonce, public/private keys, and PCR. | Integrity | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | Cryptographic key exchange, TPM, IMA |
| Phase 3 |  |  |  |  |  |  |  |  |
| 12 |  | Attester ← Verifier (2) | Attacker may attempt to modify the content of the entity. | Modifying the attestation results provided by the verifier. This breaks the integrity of RATS passport-model. | Integrity | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | Cryptographic key exchange, TPM |
| 13 |  | Attester → Relying party | Attacker may attempt to modify the content of the entity. | Attacker modifying the evidence and the results to be sent to the relying party. The replying party might be fooled to accept the given evidence, thus making a wrong decision | Integrity | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | The evidence is signed with the verifier’s cryptographic key during attestation. TPM |
| 14 |  | Attester ← Relying party (2) | Attacker may attempt to modify the content of the entity. | Modifying the decision made by the relying party and changing the deferral tickets of AWDT. Preventing it from resetting on time | Integrity | AN(Netowrk), AS(Software), AP(Priviledged) | FS4 (Integrity Assurance), FS3 (Freshness) | end-to-end encryption (TLS), AWDT |
| 15 |  | Target environment → Attestation environment (4) | Attacker may attempt to modify the content of the entity. | Modifying the decision and AWDT by the attester | Integrity | AN(Netowrk), AS(Software), AP(Priviledged) | FS4 (Integrity Assurance), FS2 (Recoverability) | end-to-end encryption (TLS), AWDT |
| Replay/Spoof |  |  |  |  |  |  |  |  |
|  |  | Where | Threat | Consequences | CIA | Attacker | Property | Mitigation |
| Phase 1 |  |  |  |  |  |  |  |  |
| 1 |  | Attester ←all→ Relying party | Attacker may attempt to replay or spoof the communication | Untrusted entities will participate in the communication, thus replay or spoof other entities | Integrity | AN(Netowrk), AP(Priviledged) |  | end-to-end encryption (TLS) communication is established with only trusted entities |
| 2 |  | Attester ← Relying party | Attacker may attempt to replay or spoof the initialization of AWDT using new or existing credentials | The attester will comply with malicious AWDT initialization, thus it will become under the attacker’s control | Integrity | AN(Netowrk), AS(Software), AP(Priviledged) |  | TLS timestamp will not allow using same time twice |
| 3 |  | Target environment → Attestation environment | Attetester replay or spoof AWDT invoking process | The attester will start with incorrect AWDT initialization, thus manipulated | Integrity | AN(Netowrk), AS(Software), AP(Priviledged) |  | TLS will detect violation from untrusted parties |
| 4 |  | RVP ← Relying party | The attacker incorrectly informs the relying party about the AWDT initialization | The relying party will miscalculate the timer and become under the attacker's control | Integrity | AN(Netowrk), AS(Software), AP(Priviledged) |  | TLS will detect violation from untrusted parties |
| 5 |  | Verifier ← RVP | Spoofing the expected state from the RVP | Attester can always mark itself as legitimate after remote attestation | Integrity | AN(Netowrk), AS(Software), AP(Priviledged) |  | The relying party configure public/private key |
| 6 |  | Attester ← RVP | Installing a malicious update and impersonating the RVP | This led the attester to be controlled by the attacker | Integrity | AN(Netowrk), AS(Software), AP(Priviledged) |  | We require for all update a signature from RVP |
| 7 |  | Target environment → Attestation environment (2) | Spoofing attester to trigger measurement | The attester will perform the measurement | Integrity | AN(Netowrk), AS(Software), AP(Priviledged) |  |  |
| Phase 2 |  |  |  |  |  |  |  |  |
| 8 |  | Attester ← Verifier | Attacker impersonating verifier and initializing attestation request | The attester will repeatedly comply with an untrusted verifier; thus, the verifier has unauthorized access to the attester’s data | Integrity | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | Nonce exchanges is done constantly to ensure freshness (Trusted) |
| 9 |  | Target environment → Attestation environment (3) | Attacker impersonates the target environment to invoke the attestation environment | This causes the attestation environment to reply to untrusted | Integrity | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | end-to-end encryption (TLS), IMA, TPM |
| 10 |  | Target environment ← Attestation environment | Attacker may attempt to spoof or replay the attester’s contents | The integrity measurement has been compromised | Integrity | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | end-to-end encryption (TLS), IMA, TPM |
| 11 |  | Attester → Vérifier | Attacker may attempt to spoof or replay the attester’s contents | Using existing communication credentials in the attester or spoofing it and reply to the verifier | Integrity | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | Nonce exchanges are done constantly to ensure freshness |
| Phase 3 |  |  |  |  |  |  |  |  |
| 12 |  | Attester ← Verifier (2) | Attacker may attempt to spoof or replay the verifier’s contents | The passport model pattern is broken; thus, it is no longer trusted | Integrity | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | Cryptographic key exchange, TPM |
| 13 |  | Attester → Relying party | Attacker may attempt to spoof or replay the attester’s contents | The attester can reproduce evidence/results and send it to the relying party to be appraised | Integrity | AN(Netowrk), AS(Software) | FS4 (Integrity Assurance), FS3 (Freshness) | Using AWDT with activation of the Deferral Ticket. |
| 14 |  | Attester ← Relying party (2) | Attacker may attempt to spoof or replay the relying party's contents | The attacker can send a wrong decision/deferral ticket to the attester. Resulting in always trusting the atester | Integrity | AN(Netowrk), AS(Software), AP(Priviledged) | FS4 (Integrity Assurance), FS3 (Freshness) | Relying party assumed to be trusted. The watchdog timer has a challenge-response protocol using HMAC when setting the timer. |
| 15 |  | Target environment → Attestation environment (4) | Attacker may attempt to spoof or replay the attestester’s contents |  | Integrity | AN(Netowrk), AS(Software), AP(Priviledged) | FS4 (Integrity Assurance), FS2 (Recoverability) |  |
| Message sequence in the RASUES protocol |  |  |  |  |  |  |  |  |
|  |  | Where | Threat | Consequences | CIA |  | Property | Mitigation |
| Phase 1 |  |  |  |  |  |  |  |  |
| 1 |  | Attester ←all→ Relying party |  |  |  |  |  |  |
| 2 |  | Attester ← Relying party |  |  |  |  |  |  |
| 3 |  | Target environment → Attestation environment |  |  |  |  |  |  |
| 4 |  | RVP ← Relying party |  |  |  |  |  |  |
| 5 |  | Verifier ← RVP |  |  |  |  |  |  |
| 6 |  | Attester ← RVP |  |  |  |  |  |  |
| 7 |  | Target environment → Attestation environment (2) |  |  |  |  |  |  |
| 8 |  | Attester ← Verifier |  |  |  |  |  |  |
| 9 |  | Target environment → Attestation environment (3) |  |  |  |  |  |  |
| 10 |  | Target environment ← Attestation environment |  |  |  |  |  |  |
| 11 |  | Attester → Vérifier |  |  |  |  |  |  |
| 12 |  | Attester ← Verifier (2) |  |  |  |  |  |  |
| 13 |  | Attester → Relying party |  |  |  |  |  |  |
| 14 |  | Attester ← Relying party (2) |  |  |  |  |  |  |
| 15 |  | Target environment → Attestation environment (4) |  |  |  |  |  |  |